Pre-embedded field-mountable optical fiber connector

FMCSC-D type

Product Introduction

For Field-mountable Optical Fiber Connectors, the SC type connectors are compliant with Telcordia GR-326-CORE. The Interface dimensions conform to industry standards and IEC standards.

Features

- SC connector type, UPC and APC polishing

- Precise alignment of V type groove

- Snap lock structure

- Low insertion loss, high return loss

- Superior repeatability, high stability and reliability

Applications

- FTTx

- Reconstruction the cable line of the computer room

Standards

- YD/T 1272.3-2015, YD/T 2341.1-2011

- IEC 61754-4 2013

- GR-1081-CORE

Specifications

| Specification | Unit | Parameter |
| --- | --- | --- |
| Cable type | mm | 3.0x2.0 Bow-type drop cable |
| Cladding diameter | μm | 125(652 & 657) |
| Fiber diameter | um | 250μm |
| Optical fiber mode | — | Single mode |
| Insertion loss | dB | ≤ 0.30(1310nm & 1550nm) |
| Return loss | dB | ≤ -50 |
| Operation time | s | 15s(Processing optical fiber time is not included) |
| Success rate | — | >98% |
| Assembly times | times | >10 |
| Bare fiber retention | N | >5 |
| Tensile strength | N | >50 |
| Operating temperature | ℃ | -40～+80 |
| Online pull test(20 N) | dB | △ IL ≤ 0.2dB |
| V groove clamping force variety(4N/30s) | dB | △ IL ≤ 0.2dB |
| Mechanical durability(500times) | dB | △ IL ≤ 0.2dB |
| 4m Drop test(3 times) | dB | △ IL ≤ 0.2dB |
| Vibration test(10~55Hz two hours in three direction) | dB | △ IL ≤ 0.2dB |
| High temperature test(85℃, 2000hours) | dB | △ IL ≤ 0.2dB |
| Temperature cycle test(-40~85℃, 500cycles) | dB | △ IL ≤ 0.2dB |
| Humidity test(85℃, 85%RH, 2000hours) | dB | △ IL ≤ 0.2dB |

Ordering information

OFC-FMC- -----

Connect Pots：SP:Single connection point,MP:Multi connection point

Fiber/Cable Type：025:250um,09:900um,20:2.0mm, 30:3.0mm,CD:3.0*2.0 Bow-type drop cable

Endface：PC,UPC,APC

Connector：FCP:FC plug,FCS:FC socket,SCP:SC plug,SCS:SC socket,LCP:LC plug,LCS:LC socket

Fiber Type：S:G.652, A1:G.657A1,A2:G.657A2, M:50/125,D62.5/125

FMC Type：MC:Mechanical type buckle locking, MS:Mechanical thread locking,F:Fusion splice type,EP:Endface pre-melted

FMC：(Field-Mountable Optical Fiber Connector)

Pre-embedded field-mountable optical fiber connector

FMCSC-PLW type

Product Introduction

For Field-mountable Optical Fiber Connectors, the SC type connectors are compliant with Telcordia GR-326-CORE. The Interface dimensions conform to industry standards and IEC standards.

Features

- SC connector type, UPC polishing

- Precise alignment of V type groove

- Thread locking structure

- Low insertion loss, high return loss

- Superior repeatability, high stability and reliability

Applications

- FTTx

- Reconstruction the cable line of the computer room

Standards

- YD/T 1272.3-2015, YD/T 2341.1-2011

- IEC 61754-4 2013

- GR-1081-CORE

Specifications

| Specification | Unit | Parameter |
| --- | --- | --- |
| Cable type | mm | 3.0x2.0 Bow-type drop cable, 2.0&3.0 indoor cable |
| Cladding diameter | μm | 125(652 & 657) |
| Fiber diameter | um | 250μm |
| Optical fiber mode | — | Single mode |
| Insertion loss | dB | ≤ 0.30(1310nm & 1550nm) |
| Return loss | dB | ≤ -50 |
| Operation time | s | 15s(Processing optical fiber time is not included) |
| Success rate | — | >98% |
| Assembly times | times | >10 |
| Bare fiber retention | N | >5 |
| Tensile strength | N | >50 |
| Operating temperature | ℃ | -40～+80 |
| Online pull test(20 N) | dB | △ IL ≤ 0.2dB |
| V groove clamping force variety(4N/30s) | dB | △ IL ≤ 0.2dB |
| Mechanical durability(500times) | dB | △ IL ≤ 0.2dB |
| 4m Drop test(3 times) | dB | △ IL ≤ 0.2dB |
| Vibration test(10~55Hz two hours in three direction) | dB | △ IL ≤ 0.2dB |
| High temperature test(85℃, 2000hours) | dB | △ IL ≤ 0.2dB |
| Temperature cycle test(-40~85℃, 500cycles) | dB | △ IL ≤ 0.2dB |
| Humidity test(85℃, 85%RH, 2000hours) | dB | △ IL ≤ 0.2dB |

Ordering information

OFC-FMC- -----

Connect Pots：SP:Single connection point,MP:Multi connection point

Fiber/Cable Type：025:250um,09:900um,20:2.0mm, 30:3.0mm,CD:3.0*2.0 Bow-type drop cable

Endface：PC,UPC,APC

Connector：FCP:FC plug,FCS:FC socket,SCP:SC plug,SCS:SC socket,LCP:LC plug,LCS:LC socket

Fiber Type：S:G.652, A1:G.657A1,A2:G.657A2, M:50/125,D62.5/125

FMC Type：MC:Mechanical type buckle locking, MS:Mechanical thread locking,F:Fusion splice type,EP:Endface pre-melted

FMC：(Field-Mountable Optical Fiber Connector)